STM32 Based Project: UART

Manual

Syed Shifat

Table of Contents

Project Features	2

- UART Communication	2

- Timer-Based Data Sending	2

- Photoresistor Reading and Threshold Detection (ADC + UART)	2

- External Interrupt Handling	2

STM32 Features Used in the Project	2

Project functions	2

Circuit diagram:	3

Pin configuration:	3

Function description	4

- 1. Function for Clock Enable	4

- 2. Function for Pin Config	4

- 3. Function for SysTick Setup	5

- 4. Function for Time Delay	5

- 5. Function for UART Config	6

- 6. Function for ADC Configuration	6

- 7. Timer Config Function	7

- 8. External Interrupt configuration function	7

- 9. All Interrupt Vector Function	8

  - USART1_IRQHandler	8

  - TIM2_IRQHandler	9

  - ADC1_2_IRQHandler	10

  - EXTI2_IRQHandler	10

- 9. Main Function	11

PCB design	11

Test Results:	12

- UART Communication Test	12

- Timer Functionality	12

- ADC (Photoresistor) Test	13

- External Interrupt Test (PA2)	13

Conclusion	13

# STM32 Based Project

This project involves communication between two STM32 microcontrollers using UART. When data is sent, the sending STM32 blinks an LED on pin PA0, and when data is received, it blinks an LED on PA1. A timer is used to send numbers from 0 to 3 every 3 seconds. On the receiving STM32, pins PA4 and PA7 are used to show the binary form of the received number, with PA4 showing the least significant bit (LSB) and PA7 showing the most significant bit (MSB). The system also reads values from a photoresistor using ADC. If the light level goes above a set high value, PA5 turns ON. If it drops below a low value, PA5 turns OFF on the other STM32. Additionally, when an external interrupt happens on pin PA2, a signal is sent to the second STM32, which then toggles an LED on PA6.

Github link: https://github.com/shifat65/3200_project_STM32_UART_ADC.git

# Project Features

## UART Communication

The STM32 communicates with another STM32 using UART.

- When data is sent, PA0 blinks.

- When data is received, PA1 blinks.

## Timer-Based Data Sending

A general-purpose timer sends data (from 0 to 3 in sequence) at regular intervals (3s). PA4 and PA7 act as indicators for the received data:

- PA4 shows the LSB (Least Significant Bit)

- PA7 shows the MSB (Most Significant Bit)

## Photoresistor Reading and Threshold Detection (ADC + UART)

The system reads values from a photoresistor.

- If the value goes above a high threshold, PA5 turns ON.

- If the value goes below a low threshold, PA5 turns OFF on the other STM32.

## External Interrupt Handling

When an external interrupt occurs on PA2, a signal is sent.

- The other STM32 responds by toggling PA6.

# STM32 Features Used in the Project

- UART

- ADC

- SysTick timer

- Timer GPIO trigger

- External interrupt

- All handled via interrupts

# Project functions

- Function for clock

  - void En_clock(void)

- Function for pin config

  - void gpio_setup(void)

- Function for systick setup

  - void systick_config(void)

- Function for time delay

  - void delay_ms(void)

  - void delay(uint32_t count)

- Function for UART config

  - void Uart1_config(void)

- Function for ADC config

  - void ADC_config(void)

- Timer config function

  - void timer_config(void)

- All interrupt vector functions

  - void TIM2_IRQHandler(void)

  - void USART1_IRQHandler(void)

  - void ADC1_2_IRQHandler(void)

  - void EXTI2_IRQHandler(void)

- Main function

  - int main(void)

# Circuit diagram:

# Pin configuration:

| Pin Name | Purpose | Config | CNF[1:0] + MODE[1:0] |
| --- | --- | --- | --- |
| PA0 | TX indication (blink on send) | GPIO Output Push-Pull | 0011 |
| PA1 | RX indication (blink on receive) | GPIO Output Push-Pull | 0011 |
| PA2 | External interrupt input | GPIO Input with Pull-up/down | 1000 |
| PA4 | Display LSB of data sent by Timer interrupt | GPIO Output Push-Pull | 0011 |
| PA5 | Output control (based on photoresistor) | GPIO Output Push-Pull | 0011 |
| PA6 | Toggle on external interrupt response | GPIO Output Push-Pull | 0011 |
| PA7 | Display MSB of data sent by Timer interrupt | GPIO Output Push-Pull | 0011 |
| PA9 | UART Transmission (USART1_TX) | Alternate Function Push-Pull | 1011 |
| PA10 | UART Reception (USART1_RX) | Input Floating | 0100 |
| PB0 | Photoresistor input (ADC) | Analog Input | 0000 |

# Function description

## 1. Function for Clock Enable

void En_clock(void)

- Enables peripheral clocks:

  - APB1ENR: USART2, TIM2

  - APB2ENR: AFIO, IOPA, IOPB, USART1, ADC1

void En_clock(void) { RCC->APB1ENR ¦= RCC_APB1ENR_USART2EN ¦ RCC_APB1ENR_TIM2EN; RCC->APB2ENR ¦= RCC_APB2ENR_AFIOEN ¦ RCC_APB2ENR_IOPAEN ¦ RCC_APB2ENR_IOPBEN ¦ RCC_APB2ENR_USART1EN ¦ RCC_APB2ENR_ADC1EN ¦ RCC_APB2ENR_ADC1EN; }

## 2. Function for Pin Config

void gpio_setup(void)

- Configures GPIO pins:

  - PA0–PA7: Various input/output configurations for UART, ADC, EXT, response signals

  - PA9: UART1 Tx → AF Output Push-Pull (1011)

  - PA10: UART1 Rx → Input floating (0100)

  - PB0: Analog input (ADC) → 0000

void gpio_setup(void) { // PA0 Tx signal for uart1:GPIO output pushpull 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF0 ¦ GPIO_CRL_MODE0); GPIOA->CRL ¦= GPIO_CRL_MODE0; // PA1 Rx signal for uart1:GPIO output pushpull 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF1 ¦ GPIO_CRL_MODE1); GPIOA->CRL ¦= GPIO_CRL_MODE1; // PA2 user input 1 GPIOA->CRL &= ~(GPIO_CRL_CNF2 ¦ GPIO_CRL_MODE2); GPIOA->CRL ¦= GPIO_CRL_CNF2_0; // PA3 user input 0; GPIOA->CRL &= ~(GPIO_CRL_CNF3 ¦ GPIO_CRL_MODE3); GPIOA->CRL ¦= GPIO_CRL_CNF3_0; // PA4 Output for USART1 receive use 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF4 ¦ GPIO_CRL_MODE4); GPIOA->CRL ¦= GPIO_CRL_MODE4; // PA5 ADC response (other stm) push pull 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF5 ¦ GPIO_CRL_MODE5); GPIOA->CRL ¦= GPIO_CRL_MODE5; // PA6 EXT interrupt response GPIO output push pull 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF6 ¦ GPIO_CRL_MODE6); GPIOA->CRL ¦= GPIO_CRL_MODE6; // PA7 Output for USART1 receive use 0011 GPIOA->CRL &= ~(GPIO_CRL_CNF7 ¦ GPIO_CRL_MODE7); GPIOA->CRL ¦= GPIO_CRL_MODE7; // PA9 UART1 Tx : Output AF push-pull 1011 GPIOA->CRH &= ~(GPIO_CRH_CNF9 ¦ GPIO_CRH_MODE9); GPIOA->CRH ¦= GPIO_CRH_CNF9_1 ¦ GPIO_CRH_MODE9; // PA10 UART1 RX : input floating 0100 GPIOA->CRH &= ~(GPIO_CRH_CNF10 ¦ GPIO_CRH_MODE10); GPIOA->CRH ¦= GPIO_CRH_CNF10_0; // PB0 analog input for ADC 0000 GPIOB->CRL &= ~(GPIO_CRL_CNF0 ¦ GPIO_CRL_MODE0); }

## 3. Function for SysTick Setup

void systick_config(void)

- SysTick LOAD = 72000-1 (for 1 ms delay at 72 MHz)

- Clock source: CPU

- Enables SysTick

void systick_config(void) { SysTick->LOAD = 72000 - 1; SysTick->VAL = 0; SysTick->CTRL = SysTick_CTRL_CLKSOURCE ¦ SysTick_CTRL_ENABLE; }

## 4. Function for Time Delay

void delay_ms(void) and void delay(uint32_t count)

- delay_ms() waits for the COUNTFLAG of SysTick

- delay(count) repeats delay_ms() for count times

void delay_ms(void) { while (!(SysTick->CTRL & SysTick_CTRL_COUNTFLAG)) { } } void delay(uint32_t count) { while (count--) { delay_ms(); } }

## 5. Function for UART Config

void Uart1_config(void)

- CR1:

  - TE (bit 3): Transmitter enable

  - RE (bit 2): Receiver enable

  - RXNEIE (bit 5): Rx not empty interrupt enable

  - UE (bit 13): USART enable

- Baud Rate: USART1->BRR = 0x1DCC (9600 for 72 MHz)

- GPIO Config: PA9 (Tx), PA10 (Rx)

- NVIC interrupt enable: USART1_IRQn

- Transmit: Write to USART1->DR

- Receive: USART1_IRQHandler handles reception using RXNE flag

void Uart1_config(void) { USART1->CR1 ¦= USART_CR1_TE ¦ USART_CR1_RE ¦ USART_CR1_RXNEIE ¦ USART_CR1_UE; // Baud rate is 9600; USART1->BRR = 0x1DCC; NVIC_EnableIRQ(USART1_IRQn); }

## 6. Function for ADC Configuration

void ADC_config(void)

- CR1:

  - AWDEN (bit 23): Analog Watchdog enable

  - AWDIE (bit 6): Analog Watchdog interrupt enable

  - AWDCH = 8 (B0 = Channel 8 → 01000)

- CR2:

  - CONT (bit 1): Continuous conversion

  - ADON (bit 0): ADC enable (called twice as per manual)

- SQR3: Set channel 8 (B0)

- HTR/LTR: High/Low threshold for watchdog

- Interrupt: ADC1_2_IRQHandler

  - SR.AWD checked

  - Action based on ADC1->DR compared to thresholds

  - AWD flag cleared after handling

void ADC_config(void) { ADC1->CR1 ¦= ADC_CR1_AWDEN ¦ ADC_CR1_AWDIE ¦ ADC_CR1_AWDCH_3; // B0 analog input pin ADC1->SQR3 = ADC_SQR3_SQ1_3; ADC1->CR2 ¦= ADC_CR2_CONT; ADC1->CR2 ¦= ADC_CR2_ADON; ADC1->HTR = 0xA00; ADC1->LTR = 0x600; delay(500); ADC1->CR2 ¦= ADC_CR2_ADON; NVIC_EnableIRQ(ADC1_2_IRQn); }

## 7. Timer Config Function

void timer_config(void)

- CNT = 0

- PSC = 7200 - 1 ( CK_CNT = 72MHz / 7200 = 10KHz)

- ARR = 50000 ( 5s period)

- DIER bit 0: Enable update interrupt

- CR1 bit 4: Downcount (DIR)

- CR1 bit 0: Timer Enable

- NVIC: TIM2_IRQn enabled

- ISR: Toggle PA0, send UART1 Sdata and increment cyclically

void timer_config(void) { TIM2->CNT = 0; TIM2->PSC = 7200 - 1; TIM2->ARR = 50000; TIM2->DIER ¦= TIM_DIER_UIE; TIM2->CR1 ¦= TIM_CR1_DIR; TIM2->CR1 ¦= TIM_CR1_CEN; NVIC_EnableIRQ(TIM2_IRQn); }

## 8. External Interrupt configuration function

- AFIO->EXTICR[1] = 0; Selecting PA as External Interrupt pin

- IMR: Unmasking.

- RTSR: Interrupt on Raising.

void Exi_config(void) { AFIO->EXTICR[1] = 0; EXTI->IMR ¦= EXTI_IMR_MR2; EXTI->RTSR ¦= EXTI_RTSR_TR2; NVIC_EnableIRQ(EXTI2_IRQn); // NVIC_EnableIRQ(EXTI3_IRQn); }

## 9. All Interrupt Vector Function

### USART1_IRQHandler

- Check RXNE

- Read from USART1->DR

- Conditional actions using GPIO depending on Rdata

- Clear RXNE flag

- Delay and reset output pin

void USART1_IRQHandler(void) { if (USART1->SR & USART_SR_RXNE) { Rdata = USART1->DR; USART1->SR &= ~USART_SR_RXNE; GPIOA->ODR ¦= GPIO_ODR_ODR1; if (Rdata == 0) { GPIOA->ODR &= ~GPIO_ODR_ODR4; GPIOA->ODR &= ~GPIO_ODR_ODR7; } else if (Rdata == 1) { GPIOA->ODR ¦= GPIO_ODR_ODR4; GPIOA->ODR &= ~GPIO_ODR_ODR7; } else if (Rdata == 2) { GPIOA->ODR &= ~GPIO_ODR_ODR4; GPIOA->ODR ¦= GPIO_ODR_ODR7; } else if (Rdata == 3) { GPIOA->ODR ¦= GPIO_ODR_ODR4; GPIOA->ODR ¦= GPIO_ODR_ODR7; } if (Rdata == 'A') { GPIOA->ODR ^= GPIO_ODR_ODR6; } if (Rdata == 'B') { GPIOA->ODR ¦= GPIO_ODR_ODR5; } else if (Rdata == 'C') { GPIOA->ODR ^= GPIO_ODR_ODR5; } delay(25); GPIOA->ODR &= ~GPIO_ODR_ODR1; } }

### TIM2_IRQHandler

- Check UIF, clear it

- Toggle PA0

- Send Sdata via UART1

- Cycle Sdata from 0 to 3

void TIM2_IRQHandler(void) { if (TIM2->SR & TIM_SR_UIF) { TIM2->SR &= ~TIM_SR_UIF; GPIOA->ODR ¦= GPIO_ODR_ODR0; USART1->DR = Sdata; delay(25); GPIOA->ODR &= ~GPIO_ODR_ODR0; Sdata++; if (Sdata == 4) { Sdata = 0; } } }

### ADC1_2_IRQHandler

- Read ADC1->DR

- If AWD:

  - Compare with HTR and LTR

  - Set present flag

  - If changed from past, send 'B' or 'C' via UART1

- Clear AWD flag at the end

void ADC1_2_IRQHandler(void) { Adc_data = ADC1->DR; if (ADC1->SR & ADC_SR_AWD) { if (Adc_data > ADC1->HTR) { present = 1; } else if (Adc_data < ADC1->LTR) { present = 0; } if ((present == 1) && (present != past)) { GPIOA->ODR ¦= GPIO_ODR_ODR0; USART1->DR = 'B'; past = present; delay(25); GPIOA->ODR &= ~GPIO_ODR_ODR0; } else if ((present == 0) && (present != past)) { GPIOA->ODR ¦= GPIO_ODR_ODR0; USART1->DR = 'C'; past = present; delay(25); GPIOA->ODR &= ~GPIO_ODR_ODR0; } ADC1->SR &= ~ADC_SR_AWD; }}

### EXTI2_IRQHandler

- Clear pending bit

- Toggle PA0

- Send 'A' via UART1

void EXTI2_IRQHandler(void) { if (EXTI->PR & EXTI_PR_PR2) { EXTI->PR ¦= EXTI_PR_PR2; } GPIOA->ODR ¦= GPIO_ODR_ODR0; USART1->DR = 'A'; delay(25); GPIOA->ODR &= ~GPIO_ODR_ODR0; }

## 9. Main Function

int main(void)

- Calls:

  - En_clock()

  - gpio_setup()

  - systick_config()

  - timer_config()

  - Uart1_config()

  - Exi_config()

  - ADC_config()

- Infinite loop: waits for interrupts to perform tasks

int main(void) { En_clock(); gpio_setup(); systick_config(); timer_config(); Uart1_config(); Exi_config(); ADC_config(); while (1) { } return 0; }

# PCB design

PCB folder contains the files of PCB design. PCB is designed using KiCad.

# Test Results:

## UART Communication Test

- Sending (PA0):

  - Every 5 seconds, a value from 0 to 3 is sent using USART1.

  - PA0 blinks shortly when data is sent (as visual indication).

- Receiving (PA1):

  - Data is received via USART1.

  - PA1 blinks shortly upon receiving any data.

- Data Decoding on Receiver STM32:

  - Data (0 to 3) sets PA4 (LSB) and PA7 (MSB) accordingly:

    - 0 → 00: PA4=LOW, PA7=LOW

    - 1 → 01: PA4=HIGH, PA7=LOW

    - 2 → 10: PA4=LOW, PA7=HIGH

    - 3 → 11: PA4=HIGH, PA7=HIGH

## Timer Functionality

- Timer2 triggers an interrupt every 3 seconds.

  - In interrupt:

    - A new value (0–3) is sent over UART.

    - PA0 blinks with each send event.

    - Value loops back to 0 after reaching 3.

## ADC (Photoresistor) Test

- Reads analog voltage on PB0.

- ADC Watchdog compares input with:

  - High Threshold (HTR): 0xA00 (~2560)

  - Low Threshold (LTR): 0x600 (~1536)

- If light increases (ADC > HTR):

  - Flag present = 1

  - Sends 'B' over UART

  - PA0 blinks (send indicator)

  - Receiver turns ON PA5

- If light decreases (ADC < LTR):

  - Flag present = 0

  - Sends 'C' over UART

  - PA0 blinks

  - Receiver toggles or turns OFF PA5

## External Interrupt Test (PA2)

- On rising edge at PA2:

  - External interrupt EXTI2_IRQHandler() is called.

  - Sends 'A' via UART.

  - PA0 blinks to indicate send.

  - Receiver toggles PA6 in response.

| Feature | Expected Result | Test Status |
| --- | --- | --- |
| UART Send/Receive | Data 0–3 sent every 3 sec, PA0/PA1 blink | Working |
| LED Pattern (PA4, PA7) | Matches received data in binary | Working |
| ADC Threshold Response | PA5 ON/OFF/toggle depending on light | Working |
| External Interrupt | Toggle PA6 on remote board on PA2 trigger | Working |
| Timer (3 sec interval) | Consistent data send every 3 sec | Working |

# Conclusion

This project successfully demonstrates UART-based communication between two STM32 microcontrollers along with additional functionality including timer-based data transmission, ADC threshold detection, and external interrupt handling. Through periodic UART transmission and reception, the system reliably transfers data and visually indicates communication events using GPIO pins (LEDs). The ADC module effectively monitors environmental light levels via a photoresistor and reacts appropriately based on defined thresholds, sending corresponding signals to the second STM32. Additionally, external interrupt support allows user interaction or sensor-triggered responses to be processed and communicated in real-time. Overall, the project integrates multiple peripherals—USART, TIM, ADC, GPIO, and EXTI—showcasing the STM32’s capabilities in embedded system design and real-time control applications.